Πανεπιστήμιο Πειραιώς

Σχολή Τεχνολογιών Πληροφορικής και Τηλεπικοινωνίων

Τμήμα Ψηφιακών Συστημάτων

Επίπεδο: Προπτυχιακό Πρόγραμμα Σπουδών

Network Security

Lab 6 Exersize

Επιβλέπον Καθηγητής: Χρήστος Ξενάκης

| Ονοματεπώνυμο | E-mail | A.M. |
| --- | --- | --- |
| Ελβις Αβντουλι | avdulielvis@gmail.com | Ε22001 |

Πειραιάς

12/12/2024

Contents

Instructions	1

Step 1: Changing Router’s hostnames	1

Step 2: ZPF	2

Step 3: ACL	3

Test: ACL	3

Step 4: AAA	5

Test: AAA	7

Instructions

(Before step 1: All commands we must run, should be run as administrator. To do that, we must type en to enter as root and then conf t to configure terminal)

Step 1: Change Router’s hostname

To change the hostname of server’s router, we must type the command hostname <name>. So, we will change router’s name as “ServerRouter”

Step 2: ZPF

The provided configuration sets up a Cisco Zone-Based Policy Firewall (ZBFW) to control traffic between two security zones, In (inside) and Out (outside). First, the security zones "In" and "Out" are defined, and two router interfaces are assigned to these zones: g0/0 to the In zone and g0/1 to the Out zone. A class map named Manage is created to match specific traffic protocols like SSH, HTTP, HTTPS, and ICMP. A policy map named Check is configured to inspect the traffic defined in the class map, enabling stateful inspection, which dynamically allows return traffic. Next, zone pairs In-Out and Out-In are created to define traffic flow between the zones. The Check policy is applied to both zone pairs, inspecting bidirectional traffic. This setup ensures controlled, inspected communication between the zones, providing firewall protection by inspecting, monitoring, and managing traffic flow while allowing only permitted protocols.

Step 3: ACL

This configuration creates an extended ACL 101 to filter ICMP and HTTP (TCP port 80) traffic between specific IP hosts. Initially, it permits ICMP traffic from 192.168.1.10, 192.168.1.20, and 192.168.1.30 to 192.168.2.100, but selectively denies TCP HTTP traffic (port 80) from those hosts. Return traffic from 192.168.2.100 to the same source hosts is explicitly permitted for both ICMP and HTTP. The ACL is then applied outbound on interface g0/0 using ip access-group 101 out. This configuration ensures that ICMP traffic is allowed while selectively blocking HTTP traffic, creating a controlled and secure flow of data.

Test: ACL

| Alice |  |
| --- | --- |
| Bob |  |
| Charlie |  |

Step 4: AAA

This configuration focuses on enabling AAA (Authentication, Authorization, and Accounting) and configuring SSH for secure remote access on a Cisco router. Initially, the enable secret password is set to secure privileged access, and the aaa new-model command is enabled to activate AAA services. Authentication is configured to use a local group, where usernames Alice, Bob, and Charlie are created with their respective passwords. The RADIUS server is set up to provide centralized authentication for users, with its IP address, port (1812), and shared secret key defined. The router domain name is set to netsec2022, and RSA keys with a modulus size of 1024 bits are generated to enable SSH functionality. The VTY lines (0–4) are configured to allow SSH access only, enforcing AAA-based login authentication. Lastly, SSH version 2 is enabled for improved security. This configuration ensures secure remote access using SSH and centralized or local AAA-based authentication.

Test: AAA